# Module "PMOD" afficheur à commande parellèle

| Référence : PMODCLP | 30.40 € HT 36.48 € TTC |
| --- | --- |

# Lextronic.fr

HD44780

http://www.youtube.com/watch?v=lQ6YKQt6Rz4

`timescale 1ns / 1ps

module LCDTest1(iClock, LCD_E, LCD_RS, LCD_RW, LCD_DB);

input  iClock;

output LCD_E;

output LCD_RS;

output LCD_RW;

output LCD_DB;

wire LCD_E;

wire LCD_RS;

wire LCD_RW;

wire[7:0] LCD_DB;

reg 		e;

reg 		rs;

reg 		rw;

reg[7:0] db;

//Responsible for putting the data on the bus

// and then pulsing the LCD_E line for 12 clocks

task WriteNybble;

input reg[3:0] nyb;

begin

db[7:4]	= nyb;

#40 	e	= 1'b1;

#12 	e	= 1'b0;

end

endtask

initial

begin

#100;	//Waiting for global reset to complete

//initialize my variables

e			=0;

rs			=0;

rw			=0;

db			=8'b0000_0000;

#750000;//Wait for LCD to come on line ~15 ms

//Do the initialization sequence.

WriteNybble(4'b0011);

#250000;//5 ms

WriteNybble(4'b0011);

#8000;//160 us

WriteNybble(4'b0011);

#8000;//160 us

WriteNybble(4'b0010);

#8000;//160 us

//Function Set  0x28

WriteNybble(4'b0010);

#50;

WriteNybble(4'b1000);

#8000;//160 us

//Entry Mode Set 0x06

WriteNybble(4'b0000);

#50;//1 us

WriteNybble(4'b0110);

#8000;//160 us

//Display On 0x0C

WriteNybble(4'b0000);

#50;//1 us

WriteNybble(4'b1100);

#8000;//160 us

//Clear display  0x01

WriteNybble(4'b0000);

#50;//1 us

WriteNybble(4'b0001);

#85000;//1.64 MS

//Set Initial DD Ram

WriteNybble(4'b1000);//sets to the first spot, upper  left

#50;//1 us

WriteNybble(4'b0000);//

#8000;//160 us

rs = 1;

#50;

//0100_0001 - write an 'A'  to the screen!!!!

WriteNybble(4'b0100);

#50;//1 us

WriteNybble(4'b0001);

#8000;//160 us

rs=0;

end

//and finally, connect the wires

assign LCD_E 	= e;

assign LCD_RS 	= rs;

assign LCD_RW	= rw;

assign LCD_DB	= db;

endmodule

http://www.cosmiac.org/pdfs/09Tutorial6.pdf

module mylcd(clk, sf_ce0, lcd_rs, lcd_rw, lcd_e, lcd_4, lcd_5, lcd_6, lcd_7);

parameter            	k = 18;

input                   clk;        // synthesis attribute PERIOD clk "50 MHz"

reg   [k+8-1:0]    	count;

output            	sf_ce0;     // high for full LCD access

reg                 	sf_ce0;

output            	lcd_e, lcd_rs, lcd_rw, lcd_7, lcd_6, lcd_5, lcd_4;

reg                 	lcd_e, lcd_rs, lcd_rw, lcd_7, lcd_6, lcd_5, lcd_4;

reg                 	lcd_busy;

reg                 	lcd_stb;

reg       [5:0]  	lcd_code;

reg       [6:0]  	lcd_stuff;

always @ (posedge clk) begin

count      <= count + 1;

lcd_busy <= 1'b1;

sf_ce0     <= 1;                 // StrataFlash Chip Enable

case (count[k+7:k+2])

0: lcd_code <= 6'h03;        // power-on initialization

1: lcd_code <= 6'h03;

2: lcd_code <= 6'h03;

3: lcd_code <= 6'h02;

4: lcd_code <= 6'h02;        // function set

5: lcd_code <= 6'h08;

6: lcd_code <= 6'h00;        // entry mode set

7: lcd_code <= 6'h06;

8: lcd_code <= 6'h00;        // display on/off control

9: lcd_code <= 6'h0C;

10: lcd_code <= 6'h00;        // display clear

11: lcd_code <= 6'h01;

12: lcd_code <= 6'h24;        // H

13: lcd_code <= 6'h28;

14: lcd_code <= 6'h26;        // e

15: lcd_code <= 6'h25;

16: lcd_code <= 6'h26;        // l

17: lcd_code <= 6'h2C;

18: lcd_code <= 6'h26;        // l

19: lcd_code <= 6'h2C;

20: lcd_code <= 6'h26;        // o

21: lcd_code <= 6'h2F;

22: lcd_code <= 6'h22;        //

23: lcd_code <= 6'h20;

24: lcd_code <= 6'h25;        // W

25: lcd_code <= 6'h27;

26: lcd_code <= 6'h26;        // o

27: lcd_code <= 6'h2F;

28: lcd_code <= 6'h27;        // r

29: lcd_code <= 6'h22;

30: lcd_code <= 6'h26;        // l

31: lcd_code <= 6'h2C;

32: lcd_code <= 6'h26;        // d

33: lcd_code <= 6'h24;

34: lcd_code <= 6'h22;        // !

35: lcd_code <= 6'h21;

default: lcd_code <= 6'h10;

endcase

lcd_stb <= ^count[k+1:k+0] & ~lcd_rw & lcd_busy;

lcd_stuff <= {lcd_stb,lcd_code};

{lcd_e,lcd_rs,lcd_rw,lcd_7,lcd_6,lcd_5,lcd_4} <= lcd_stuff;

end

endmodule

NET "lcd_e" LOC = "M18";
NET "lcd_rs" LOC = "L18";
NET "lcd_rw" LOC = "L17";
NET "sf_d8" LOC = "R15";
NET "sf_d9" LOC = "R16";
NET "sf_d10" LOC = "P17";
NET "sf_d11" LOC = "M15";
NET "sf_ce0" LOC = "D16";
NET "clk" LOC = "C9";

Interfacing

F = 50 Mhz	donc T = 20ns

Tempo de 15ms :

Tempo de 4.1ms :

Tempo de 100us :

Tempo de 40us :

Enable : il faut maintenir à l’état haut pendant au moins 230ns.

Si on prend 240 ns , on obtient 12 cycles d’horloge 12 * 20ns = 240ns

Initialisation de l’afficheur en mode 8 bits

WAIT 15ms

| RS | R/W | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | * | * | * | * |

0x30 avec enable = 1 pendant 240ns

WAIT 4.1ms

| RS | R/W | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | * | * | * | * |

0x30 avec enable = 1 pendant 240ns

WAIT 100us

| RS | R/W | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | * | * | * | * |

0x30 avec enable = 1 pendant 240ns

WAIT 4.1ms

| RS | R/W | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | * | * | * | * |

avec enable = 1 pendant 240ns

http://nalhossri.free.fr/LCD4bits.h/Fonctions_LCD4bits.h.pdf

http://www.eng.utah.edu/~cs3710/slides/OtherIOx6.pdf

http://www.xilinx.com/support/documentation/application_notes/xapp904.pdf