module moore_3processes(input clk, input reset, inp

ut x, output reg parity);

reg state, nextstate;

parameter S0=0, S1=1;

always @(posedge clk or posedge reset)

if (reset)

state <= S0;

else

state <= nextstate;

always @(state) begin // always block to compute output

case(state)

S0: parity = 0;

S1: parity = 1;

endcase

end

always @(state or x) // always block to compute

nextstate

begin

nextstate = S0;

case(state)

S0: if(x)

nextstate = S1;

S1: if(!x)

nextstate = S1;

endcase

end

endmodule

https://pdfs.semanticscholar.org/b4d2/38899ab8ac75a271d3dcc07b595867946b7a.pdf

http://www.sunburst-design.com/papers/CummingsSNUG2000Boston_FSM.pdf

1 seul process

module fsm (clk, reset, x1, outp);

input clk, reset, x1;

output outp;

reg outp;

reg [1:0] state;

parameter s1 = 2'b00; parameter s2 = 2'b01;

parameter s3 = 2'b10; parameter s4 = 2'b11;

always@(posedge clk or posedge reset)

begin

if (reset)

begin

state = s1; outp = 1'b1;

end

else

begin

case (state)

s1: begin

if (x1==1'b1) state = s2;

else          state = s3;

outp = 1'b1;

end

s2: begin

state = s4; outp = 1'b1;

end

s3: begin

state = s4; outp = 1'b0;

end

s4: begin

state = s1; outp = 1'b0;

end

endcase

end

end

endmodule

Avec 2 process

module fsm (clk, reset, x1, outp);

input clk, reset, x1;

output outp;

reg outp;

reg [1:0] state;

parameter s1 = 2'b00; parameter s2 = 2'b01;

parameter s3 = 2'b10; parameter s4 = 2'b11;

always @(posedge clk or posedge reset)

begin

if (reset)

state = s1;

else

begin

case (state)

s1: if (x1==1'b1) state = s2;

else          state = s3;

s2: state = s4;

s3: state = s4;

s4: state = s1;

endcase

end

end

always @(state)

begin

case (state)

s1: outp = 1'b1;

s2: outp = 1'b1;

s3: outp = 1'b0;

s4: outp = 1'b0;

endcase

end

endmodule

Avec 3 process

module fsm (clk, reset, x1, outp);

input clk, reset, x1;

output outp;

reg outp;

reg [1:0] state;

reg [1:0] next_state;

parameter s1 = 2'b00; parameter s2 = 2'b01;

parameter s3 = 2'b10; parameter s4 = 2'b11;

always @(posedge clk or posedge reset)

begin

if (reset)  state = s1;

else        state = next_state;

end

always @(state or x1)

begin

case (state)

s1: if (x1==1'b1) next_state = s2;

else          next_state = s3;

s2: next_state = s4;

s3: next_state = s4;

s4: next_state = s1;

endcase

end

always @(state)

begin

case (state)

s1: outp = 1'b1;

s2: outp = 1'b1;

s3: outp = 1'b0;

s4: outp = 1'b0;

endcase

end

endmodule

http://asic.co.in/Index_files/verilogexamples.htm#link1

https://www.slideshare.net/PaulLaskowski1/verilog-hdl-training-course

https://learn.digilentinc.com/Documents/263

https://learn.digilentinc.com/classroom/tutorials/

// Verilog code
module test001(clk, pulse, clkout);

input clk;
input pulse;
output clkout;

reg cnt, temp;
initial begin cnt = 0;
temp = 0;
end

always @(posedge clk)
if(pulse && !cnt) begin
cnt <= ~cnt;
temp <= 1;
end
else if(!pulse) begin
cnt <= 0;
end
else
temp <= 0;


assign clkout = temp|clk;

endmodule

// edge detect module

module edge_detect(

input  CLK,

input  RST,

input  IN,

output OUT

);

reg a, b;

// the edge detect signal is (b AND (NOT a))

assign OUT = a & !b;

always @(posedge CLK) begin

// It's always good to have a reset condition, otherwise

// the state of the register will show up as undertemined

// in simulation ('x')

if (RST == 1'b1) begin

a <= 0;

b <= 0;

end

else begin

a <= IN;

b <= a;

end

end

endmodule

edge detector: rising

edge detector: failing