Verilog PLI (Programming Language Interface) is a mechanism to invoke C or C++ functions from Verilog code.

PLI is an application program interface (API) to your simulator environment. PLI routines let you write applications that create new simulator system tasks and manipulate instantiated simulation objects contained in a Verilog HDL design. You integrate your PLI application with the simulator to create a customized version of the simulator that contains the new simulator system tasks.

PLI consists of a set of access and utility routines that you call from standard C programming language functions. The PLI routines interact with instantiated simulation objects created in Verilog HDL designs. Instantiated simulation objects are individually accessible instances of low-level modules that are contained in higher-level modules.

For example, if a module named flipflop contains wire q and is instantiated in module shifter as ff1 and ff2, then shifter.ff1.q and shifter.ff2.q are two distinct objects, each with its own set of accessible values and properties. Using the PLI access routines, you can get information from a design about each of these separate instances.

The function invoked in Verilog code is called a system call.

$display("hello world");

The makefile compiles and links your application

Le Verilog (HARDWARE DESCRIPTION LANGUAGE - HDL) est un langage de description matériel utilisé pour décrire  un système numérique. Par exemple, un commutateur de réseau, un microprocesseur
ou d'un mémoire ou une bascule. Cela signifie simplement que, en utilisant un HDL on peut décrire tout
matériel (numérique) quelque soit son niveau de complexité.

Niveau de représentation, on parle d’abstraction :

Niveau comportementale

Niveau RTL

Niveau porte

Niveau transistor

Flot de conception :

- Spécification

- Conception haut niveau

- Conception fonctionnel

- Codage RTL

- Simulation

- Synthèse

- Placement / routage

- Post simulation

Les conventions lexicales et sémantiques sont identiques au langage C.

Sensibles à la case

Notion de module pour définir un bloc hiérarchique qui effectue un traitement. Les ports constitues les entrées –sorties du module qui permettent de communiquer avec les autres module du circuit.

Lorsque l’on utilise un module, on parle d’instanciation (implicite ou explicite).

Types de données :

- net : connexion entre des elements -> wire

- reg : variable utilisé pour stocker une donnée