Etape successive:

Floor & power planning

Placement

Clocks Tree synthesis

Routing

Timing analysis

Verification & calcul de le puissance

//////////////////////////////////////////////////////////////////////////////////////////////

Avant de passer à l’étape de conception physique du circuit il est nécessaire :

- Lancer la synthése pour obtenir une netlist comprenent les portes (gate level)

- Vérifier que le niveau gate level rempli les fonctionnalitées (simulation)

- Générer le fichier de contrainte des delais  sdc (standard delay contraints)

Design import :

- LEF , layer, via définition

- LIB (TLF), delay, capacitance

- Netlist verilog

- SDC

- DEF

Placement : Two Associated Cost functions

- Reduce Total wiring or routing length

- Distribute standard cell instances homogeneously

in ASIC Core such that optimal equilibrium among

vertical and horizontal routing is achieved

Detailed Placement

-After global placement

To Refine placement based on congestion, timing and

power

-Congestion Driven Placement

To distance standard cell instances from each other such

that more routing tracks are created between them

-Timing Driven Placement

To optimize large sets of path delays

- Net Based

Try to control the delay on signal path by imposing an

upper bound delay or weight to net

Power distribution :

- Stripes

- Rails

- Clock Tree Synthesis

- Automatic insertion of buffers/inverters along

- the clock path to balance the clock delay to all the clock inputs.

- Why balancing clock delay?

- Clock Skew?