# NCVxxxxx Digital Verification Test Plan

# 1: I2C Regs

## 1.1 I2C defaults

DESCRIPTION:

Verify defaults of I2C registers are in line with specified ones.

Test sequence:

- Perform reset

- Force OTP config not fused

- Start chip (POR release) and make EN pin high

- Enter in test mode

- Read all I2C registers and check the read value is equal to PTS expected value

TESTCASE:  default.v

## 1.2 I2C Product ID

DESCRIPTION:

Verify Product ID.

Test sequence:

- Perform reset

- Start chip by rising AVIN and make EN pin high

- Enter in test mode.

- Force all possible PID values at DigTop input

- Read corresponding value in PID I2C register.

TESTCASE: 	test_outputs.v

## 1.3 I2C Revision ID

DESCRIPTION:

Verify Revision ID.

Test sequence:

- Perform reset

- Start chip by rising AVIN and make EN pin high

- Enter in test mode

- Force all possible RID values at DigTop input

- Read corresponding value in RID I2C register.

TESTCASE: 	test_outputs.v

# 2: FSM

## 2.1 UVLO

DESCRIPTION:

Verify impact of uvlo event in o_buck_en, PG, and comparator enables outputs in different states and configurations of tb_uvlo_and_tsd_blk test bit. Buck needs to be turned off asynchronously with uvlo riding.

Test sequence:

- Perform reset

- Start chip by rising AVIN and make EN pin high

- Wait PG

- Check that uvlo and tsd comparators enable and o_buck_en outputs are equal to 1.

- Generate an uvlo event by dropping AVIN level

- Check time between the end of uvlo event and o_buck_en negedge (0-100ns).

- Wait Power failed

- Check that o_VaUvlo_comp_en is equal to 1 and the low state of buck_en and o_therm_comp_en outputs

- Exit uvlo state by raising AVIN level

- Check time between the end of uvlo event and when o_buck_en is high (0-10us).

- Check that uvlo and tsd comparators enable and o_buck_en outputs are equal to 1.

- Enter in test mode

- Set tb_uvlo_and_tsd_blk to 1 by I2C write to ignore uvlo event

- Exit test mode

- Generate an uvlo event by dropping AVIN level

- Check that uvlo and tsd comparators enable and o_buck_en outputs are equal to 1

- Perform reset by dropping AVIN and EN pin to 0

- Start chip by rising AVIN and make EN pin high

- Generate tsd event

- Wait PG failed

- Check that o_VaUvlo_comp_en  and o_therm_comp_en  are equal to 1 and o_buck_en is equal to 0

- Generate uvlo event

- Check that o_VaUvlo_comp_en  is equal to 1 and o_buck_en and o_therm_comp_en  are equal to 0

TESTCASE: 	uvlo.v

## 2.2 Power Good

DESCRIPTION:

Verify impact of different states and i_pg_buck on PG pin (o_rstb_pg_b)

Test sequence:

- Raise AVIN

- Check power is failed : o_rstb_pg_b is high

- Make EN pin high

- Wait power good

- Set i_pg_buck to 0

- Check o_rstb_pg_b is high

- Set i_pg_buck to 1

- Check o_rstb_pg_b is low

- Generate tsd event by raising temperature level

- Check o_rstb_pg_b is high

- Exit tsd event by dropping temperature level

- Check o_rstb_pg_b is low

- Generate uvlo event by dropping uvlo level

- Check o_rstb_pg_b is high

- Exit tsd event by raising Avin level

- Check o_rstb_pg_b is low

- Make EN pin low

- Check o_rstb_pg_b is high

TESTCASE: 	pg.v

# 3: OTP control

## 3.1 OTP load

DESCRIPTION:

Check Load OTP bits

Test sequence:

- Force OTP bits

- Perform reset

- Start chip by rising AVIN

- Raise EN pin

- Enter Test Mode

- Read out all I2C TRIM registers and compare with expected data Write for each address in I2C TRIM registers

TESTCASE: 	read_otp.v

## 3.2 OTP fuse

DESCRIPTION:

Check fused, load, emulation and reload otp mechanisms available in test mode thanks to I2C test bits.

Test sequence:

- Perform reset

- Start chip by rising AVIN

- Raise EN pin

- Enter Test Mode

- Read out all I2C TRIM registers and compare with expected default values 8’h00 (OTP is not fused)

- Write 8’hAA in all I2C TRIM registers

- Fuse first thirty two OTP bits by setting tb_otp_addr_sel_32b test bits to 0 and otp_prog_req to 1

- Read out all I2C TRIM registers and compare with expected values write by I2C during step 12

- Perforn reset and wait OTP load

- Enter Test Mode

- Read out first four I2C TRIM registers (32bits) and compare with fused values during step 13

- Read out last four I2C TRIM registers (32bits) and compare with expected default values 8’h00.

- Write 8’h55 in all I2C TRIM registers

- Read out first four I2C TRIM registers (32bits) and compare with 8’hFF value (or operation between fused values during step 13 and writed values during step 19)

- Read out last four I2C TRIM registers (32bits) and compare with values 8’h55 write in I2C TRIM registers during step 19

- Enable zero emulation by setting tb_otp_emul_zero bit to 1

- Write 8’h55 in all I2C TRIM registers

- Read out all I2C TRIM registers and compare with values 8’h55

- Fuse all OTP registers thantks to tb_otp_addr_sel_32b and tb_otp_prog_req test bits

- Disable tb_otp_emul_zero test bit

- Reload OTP by setting tb_otp_read_req

- Read out first four I2C TRIM registers (32bits) and compare with 8’hFF values (result of or operation between fused values during step 13 and during step 25)

- Read out last four I2C TRIM registers (32bits) and compare with 8’h55 values fused during step 25

TESTCASE: 	fuse_otp.v

# 4: ADCs

## 4.1 blabla

DESCRIPTION:

Linearity .

Test sequence:

- Perform reset

- Force OTP config not fused

- Start chip (POR release) and make EN pin high

- Enter in test mode

- Read all I2C registers and check the read value is equal to PTS expected value

TESTCASE:  default.v